# =============================================================================

# Pybricks (SPIKE Prime / Inventor Hub)

# WRO Obstacle Challenge — NO parallel parking

# -----------------------------------------------------------------------------

# Merge of:

#   * WRO Open Challenge corridor driver (90 deg gyro arcs at openings)

#   * HuskyLens color dodges (GREEN = LEFT, RED = RIGHT)

#

# PORT MAP

#     A = drive motor (rear, with encoder)

#     B = steering motor (front wheels)

#     C = distance sensor LEFT

#     D = distance sensor FRONT

#     E = distance sensor RIGHT

#     F = HuskyLens UART, Color Recognition, 9600 baud

#

# Teach the HuskyLens:

#     Color ID 1 = GREEN pillar  -> pass on the LEFT

#     Color ID 2 = RED   pillar  -> pass on the RIGHT

#

# WHAT THE CAR DOES

#     CRUISE     wall-center + IMU heading hold, watch for openings AND colors

#     COLOR      GREEN: 35 deg LEFT while the pillar is visible, then hold

#                50 deg RIGHT for 30 cm, straighten the wheels, then go

#                20 cm straight. RED is the mirror: 35 deg RIGHT, then

#                50 deg LEFT for 30 cm, straighten, 20 cm straight.

#                Corridor heading is KEPT (gyro is not zeroed) so the next

#                90 deg corner still lines up. Wall-centering is boosted

#                briefly so the car returns to the middle of the lane.

#     APPROACH   opening seen; hold heading until the front wall is close

#     TURN       closed-loop 90 deg gyro arc (never stops mid-run)

#     POST       short gyro-straight, then back to CRUISE

#     FINAL      after MAX_TURNS (12 = 3 laps of 4 corners), center for

#                FINAL_DRIVE_MM and stop. No parking.

#

# The car is not allowed to stop until every 90 deg turn is done.

# A corner turn is ONLY started when a real opening is detected (the open

# side reads farther than OPEN_DISTANCE_MM = 1000 mm) AND the front wall

# has been reached. A front wall alone -- e.g. a color block that the

# HuskyLens has not identified yet -- NEVER triggers a turn; the car just

# slows down and lets the color dodge handle it.

#

# STARTUP

#     1. Point the car straight down the corridor.

#     2. Physically straighten the front wheels.

#     3. Press Run. That heading becomes 0.

# =============================================================================

from pybricks.hubs import PrimeHub

from pybricks.pupdevices import Motor, UltrasonicSensor

from pybricks.parameters import Port, Stop, Icon, Color

from pybricks.tools import wait, StopWatch

from pybricks.iodevices import UARTDevice

# ------------------------- PORTS -------------------------

hub = PrimeHub()

drive = Motor(Port.A)

steer = Motor(Port.B)

left_sensor = UltrasonicSensor(Port.C)

front_sensor = UltrasonicSensor(Port.D)

right_sensor = UltrasonicSensor(Port.E)

# --------------------- DRIVE SETTINGS -------------------

# Regulated rear-motor speed, deg/s. Negative if the car drives backward.

DRIVE_SPEED = -500

TURN_SPEED = -280           # slower in the 90 deg arc (same SIGN as DRIVE)

STEERING_SPEED = 400       # deg/s of the steering servo

# Steering MOTOR angle, not the road-wheel angle.

MAX_STEERING_DEG = 32

MAX_TURN_STEERING_DEG = 40          # also the hard clamp for dodges/recovery

# STEERING_SIGN: +1 if a POSITIVE motor angle turns the car LEFT

# (same meaning as the working SPIKE 3 program). Flip to -1 if reversed.

STEERING_SIGN = 1

# YAW_SIGN: used after converting Pybricks heading (clockwise +)

# into a left-positive yaw like the SPIKE 3 program. Flip if turns go

# the wrong way.

YAW_SIGN = 1

# ---------------------- STOP SETTINGS -------------------

# Front-wall stopping is ONLY used in FINAL, after all MAX_TURNS are done.

STOP_DISTANCE_MM = 180

FRONT_CONFIRMATIONS = 2

FRONT_STOP_MAX_YAW_DEG = 12

EMERGENCY_STOP_DISTANCE_MM = 10

# ------------------- CRUISE CONTROLLER GAINS ------------

WALL_KP = 0.11

WALL_DEADBAND_MM = 8

WALL_FILTER_AMOUNT = 0.25

HEADING_KP = 1.20

YAW_DAMPING = 1.80

# After a color dodge, wall-centering is stronger for this long so the

# car hunts the middle of the lane instead of staying offset.

RECENTER_MS = 3000

RECENTER_WALL_KP = 0.20

RECENTER_DEADBAND_MM = 4

# ------------------- TURN DETECTION ---------------------

SPIKE_JUMP_MM = 500

SPIKE_CONFIRMATIONS = 4

BASELINE_FILTER_AMOUNT = 0.12

MAX_WALL_DISTANCE_MM = 2000

# An opening ONLY counts when the open side reads FARTHER than this.

# The GREEN/RED color blocks stand INSIDE the corridor, so the side

# sensors never read > 1000 mm for them -- a block can never look like

# an opening, and a front wall without a real opening never triggers a

# turn (the HuskyLens color dodge handles blocks instead).

OPEN_DISTANCE_MM = 1000

# ------------------- TURN EXECUTION ---------------------

TURN_AT_FRONT_DISTANCE_MM = 900

FRONT_CLOSE_CONFIRMATIONS = 1

WALL_RETURN_CONFIRMATIONS = 2

WALL_RETURN_MARGIN_MM = 150     # side wall counts as "back" if within baseline + this

MAX_APPROACH_DISTANCE_MM = 700

POST_TURN_DRIVE_MM = 100

ODOMETER_MM_PER_MOTOR_DEG = 0.49    # ~56 mm wheel; pi*56/360

TURN_ANGLE_DEG = 90

TURN_KP = 2.0

TURN_DONE_TOLERANCE_DEG = 6

TURN_TIMEOUT_MS = 6000

MAX_TURNS = 12                      # 3 laps x 4 corners

FINAL_DRIVE_MM = 300                # settle between walls, then stop

# When something is straight ahead but NO opening was detected, never turn:

# it is almost certainly a color block the HuskyLens has not identified yet.

# Slow down so the camera has time to see the color, and the color dodge

# takes over from there.

BLOCK_AHEAD_SLOWDOWN_MM = 500

BLOCK_AHEAD_SPEED = -200

# ------------------- COLOR (HuskyLens) ------------------

#  GREEN  -> pass on the LEFT   (dodge LEFT, recover RIGHT)

#  RED    -> pass on the RIGHT  (dodge RIGHT, recover LEFT)

#  Logical steering convention here: + = wheels RIGHT, - = wheels LEFT.

GREEN_ID = 1                        # color ID of the GREEN pillar

RED_ID = 2                          # color ID of the RED pillar

GREEN_STEER = -35                   # -35 logical  = LEFT dodge

GREEN_RECOVER = 50                  # +50 logical  = RIGHT recovery

RED_STEER = 35                      # +35 logical  = RIGHT dodge

RED_RECOVER = -50                   # -50 logical  = LEFT recovery

COLOR_RECOVER_MM = 300              # 30 cm HOLDING the recovery angle

RECOVER_STRAIGHT_MM = 200           # 20 cm straight after straightening

COLOR_LOST_CONFIRM_MS = 300

HUSKY_PORT = Port.F

HUSKY_BAUD = 9600

HUSKY_POWER_PIN = 0                 # 2 if the camera needs hub pin power

HUSKY_TIMEOUT_MS = 60

LOOP_TIME_MS = 50

PRINT_DEBUG = False

# ------------------------ MODES -------------------------

MODE_CRUISE = 0

MODE_APPROACH = 1

MODE_TURN = 2

MODE_POST = 3

MODE_FINAL = 4

MODE_NAMES = ("CRUISE", "APPROACH", "TURN", "POST", "FINAL")

# ------------------------------------------------------------------ helpers

def clamp(value, low, high):

if value < low:

return low

if value > high:

return high

return value

def distance_is_valid(value):

return value is not None and value > 0

def read_mm(sensor):

"""Pybricks returns 2000 mm when nothing is seen; map that to -1."""

try:

d = sensor.distance()

except OSError:

return -1

if d is None or d <= 0 or d >= 2000:

return -1

return d

def angle_difference(new_angle, old_angle):

difference = new_angle - old_angle

while difference > 180:

difference -= 360

while difference < -180:

difference += 360

return difference

def wrap_angle(angle):

while angle > 180:

angle -= 360

while angle <= -180:

angle += 360

return angle

def yaw_left_positive():

"""

Pybricks hub.imu.heading() is clockwise-positive (RIGHT).

The Open Challenge controller is left-positive. Convert here so the

rest of the math matches the working SPIKE 3 program.

"""

return wrap_angle(YAW_SIGN * (-hub.imu.heading()))

def traveled_mm(start_position):

return abs(drive.angle() - start_position) * ODOMETER_MM_PER_MOTOR_DEG

def update_side_reading(reading_mm, baseline, open_count, open_threshold_mm):

"""

Wall tracking for one side. An 'open' count only builds up when the

side truly opens up: the reading jumps away from the wall baseline AND

is farther than open_threshold_mm (1000 mm). A missing reading means

no wall within 2 m -- that is open too. Wall-like readings (a color

block in the corridor) always stay below the threshold and never count.

"""

if distance_is_valid(reading_mm):

if baseline is None:

if reading_mm <= MAX_WALL_DISTANCE_MM:

baseline = reading_mm

return baseline, 0

if (

reading_mm > baseline + SPIKE_JUMP_MM

and reading_mm > open_threshold_mm

):

return baseline, open_count + 1

baseline = baseline + (reading_mm - baseline) * BASELINE_FILTER_AMOUNT

return baseline, 0

if baseline is not None:

return baseline, open_count + 1

return baseline, 0

def u16le(data, index):

return data[index] | (data[index + 1] << 8)

# ------------------------------------------------------------------ motors

def start_drive(speed=None):

drive.run(DRIVE_SPEED if speed is None else speed)

def stop_drive():

drive.brake()

def apply_steering(conceptual_left_deg, last_target, limit=None):

"""

conceptual_left_deg: + = turn LEFT (SPIKE 3 convention).

STEERING_SIGN maps that onto the motor.

"""

if limit is None:

limit = MAX_TURN_STEERING_DEG

target = int(STEERING_SIGN * conceptual_left_deg)

target = clamp(target, -limit, limit)

if target != last_target:

steer.run_target(STEERING_SPEED, target, then=Stop.HOLD, wait=False)

return target

def steer_logical(logical_right_deg, wait_done=True):

"""logical_right_deg: + = wheels RIGHT, - = wheels LEFT."""

# conceptual_left = -logical_right, then STEERING_SIGN

motor_angle = STEERING_SIGN * (-logical_right_deg)

motor_angle = clamp(motor_angle, -MAX_TURN_STEERING_DEG, MAX_TURN_STEERING_DEG)

steer.run_target(STEERING_SPEED, motor_angle, then=Stop.HOLD, wait=wait_done)

# ------------------------------------------------------------------ HuskyLens

CMD_KNOCK = bytes((0x55, 0xAA, 0x11, 0x00, 0x2C, 0x3C))

CMD_BLOCKS_LEARNED = bytes((0x55, 0xAA, 0x11, 0x00, 0x24, 0x34))

CMD_OK = 0x2E

CMD_INFO = 0x29

CMD_BLOCK = 0x2A

def _checksum(payload):

s = 0

for b in payload:

s += b

return s & 0xFF

def cmd_blocks_by_id(color_id):

frame = bytearray((0x55, 0xAA, 0x11, 0x02, 0x27, color_id & 0xFF, 0x00))

frame.append(_checksum(frame))

return bytes(frame)

class HuskyLens:

def __init__(self, port, baudrate=9600, power_pin=0):

kwargs = {"baudrate": baudrate, "timeout": 200}

if power_pin:

kwargs["power_pin"] = power_pin

self.uart = UARTDevice(port, **kwargs)

wait(300)

self.uart.clear()

def _read_exact(self, n, timeout_ms):

watch = StopWatch()

data = b""

while len(data) < n and watch.time() < timeout_ms:

waiting = self.uart.waiting()

if waiting:

take = min(waiting, n - len(data))

data += self.uart.read(take)

else:

wait(1)

return data

def _read_packet(self, timeout_ms):

watch = StopWatch()

while watch.time() < timeout_ms:

hdr = self._read_exact(2, max(5, timeout_ms - watch.time()))

if len(hdr) < 2:

return None, b""

if hdr[0] == 0x55 and hdr[1] == 0xAA:

rest = self._read_exact(3, 40)

if len(rest) < 3:

return None, b""

address, length, cmd = rest[0], rest[1], rest[2]

if address != 0x11:

continue

payload = self._read_exact(length + 1, 40)

if len(payload) < length + 1:

return None, b""

return cmd, payload[:length]

if hdr[1] == 0x55:

extra = self._read_exact(1, 10)

if extra == b"\xAA":

rest = self._read_exact(3, 40)

if len(rest) == 3 and rest[0] == 0x11:

length, cmd = rest[1], rest[2]

payload = self._read_exact(length + 1, 40)

if len(payload) >= length + 1:

return cmd, payload[:length]

return None, b""

def knock(self):

try:

self.uart.clear()

self.uart.write(CMD_KNOCK)

cmd, _ = self._read_packet(150)

return cmd == CMD_OK

except OSError:

return False

def seen_ids(self, wanted=(1, 2)):

found = set()

try:

self.uart.clear()

self.uart.write(CMD_BLOCKS_LEARNED)

cmd, payload = self._read_packet(HUSKY_TIMEOUT_MS)

if cmd != CMD_INFO or len(payload) < 2:

return self._seen_by_id(wanted)

n = u16le(payload, 0)

for _ in range(n):

bcmd, bpay = self._read_packet(40)

if bcmd != CMD_BLOCK or len(bpay) < 10:

break

block_id = u16le(bpay, 8)

if block_id in wanted:

found.add(block_id)

return found

except OSError:

return found

def _seen_by_id(self, wanted):

found = set()

for color_id in wanted:

try:

self.uart.clear()

self.uart.write(cmd_blocks_by_id(color_id))

cmd, payload = self._read_packet(HUSKY_TIMEOUT_MS)

if cmd == CMD_INFO and len(payload) >= 2 and u16le(payload, 0) > 0:

found.add(color_id)

self._read_packet(40)

except OSError:

pass

return found

# ------------------------------------------------------------------ color dodge

def drive_while_color_visible(husky, color_id, logical_steer):

"""Hold 45 deg and keep driving until that color has been gone a moment."""

steer_logical(logical_steer, wait_done=False)

start_drive()

lost = StopWatch()

while True:

steer_logical(logical_steer, wait_done=False)

start_drive()

if color_id in husky.seen_ids((color_id,)):

lost.reset()

elif lost.time() >= COLOR_LOST_CONFIRM_MS:

break

wait(10)

def drive_mm_while_steering(distance_mm, steer_start, steer_end):

"""Drive distance_mm, slewing steering from start to end. Keep rolling.

Pass steer_start == steer_end to HOLD a constant angle for the distance."""

target_motor = distance_mm / ODOMETER_MM_PER_MOTOR_DEG

if target_motor < 1:

return

start_ang = drive.angle()

steer_logical(steer_start, wait_done=True)

start_drive()

last_logical = None

while abs(drive.angle() - start_ang) < target_motor:

travelled = abs(drive.angle() - start_ang)

progress = travelled / target_motor

if progress > 1:

progress = 1

logical = steer_start + (steer_end - steer_start) * progress

if logical != last_logical:

steer_logical(logical, wait_done=False)

last_logical = logical

wait(10)

def color_dodge(husky, color_id, first_steer, recover_steer):

"""Dodge at first_steer while the color is visible, then recover.

GREEN: first_steer=-35 (LEFT),  recover_steer=+50 (RIGHT)

RED:   first_steer=+35 (RIGHT), recover_steer=-50 (LEFT)

Sequence: hold the recovery angle for COLOR_RECOVER_MM (30 cm),

straighten the wheels, then drive RECOVER_STRAIGHT_MM (20 cm)

straight. The car never stops."""

drive_while_color_visible(husky, color_id, first_steer)

drive_mm_while_steering(COLOR_RECOVER_MM, recover_steer, recover_steer)

steer_logical(0, wait_done=True)            # straighten tires, keep rolling

drive_mm_while_steering(RECOVER_STRAIGHT_MM, 0, 0)

start_drive()

# ------------------------------------------------------------------ main run

def drive_course(husky, connected):

"""

One full attempt. Returns only after MAX_TURNS + FINAL_DRIVE_MM

(or a FINAL-phase front wall). Never returns mid-run.

"""

reference_yaw = yaw_left_positive()

corridor_yaw = reference_yaw          # current corridor heading (kept, never zeroed)

filtered_wall_error = 0.0

previous_yaw_deg = reference_yaw

last_steering_target = 0

close_front_count = 0

baseline_left = None

baseline_right = None

open_count_left = 0

open_count_right = 0

mode = MODE_CRUISE

turn_direction = 0

turn_target_yaw = reference_yaw

turn_watch = StopWatch()

phase_start_position = drive.angle()

turn_side_baseline = None

front_close_count = 0

wall_return_count = 0

turns_completed = 0

green_latched = False

red_latched = False

clock = StopWatch()

recenter_until = 0

print("Reference yaw: %.1f deg" % reference_yaw)

start_drive()

while True:

loop_watch = StopWatch()

yaw_deg = yaw_left_positive()

yaw_change = angle_difference(yaw_deg, previous_yaw_deg)

previous_yaw_deg = yaw_deg

heading_error = angle_difference(yaw_deg, corridor_yaw)

front_mm = read_mm(front_sensor)

left_mm = read_mm(left_sensor)

right_mm = read_mm(right_sensor)

all_turns_done = turns_completed >= MAX_TURNS

# ---- FINAL-only stop (cannot fire before all 90 deg turns) ----

if mode == MODE_FINAL and all_turns_done:

emergency_stop = (

distance_is_valid(front_mm)

and front_mm <= EMERGENCY_STOP_DISTANCE_MM

)

normal_front_wall = (

distance_is_valid(front_mm)

and front_mm <= STOP_DISTANCE_MM

and abs(heading_error) <= FRONT_STOP_MAX_YAW_DEG

)

if normal_front_wall:

close_front_count += 1

else:

close_front_count = 0

if emergency_stop or close_front_count >= FRONT_CONFIRMATIONS:

stop_drive()

steer_logical(0, wait_done=True)

print("Final approach front wall - stopping")

return

else:

close_front_count = 0

# ---- GREEN / RED pillars (CRUISE only) ----

# Do this BEFORE opening detection so a pillar is never mistaken

# for a corner. Corridor heading (corridor_yaw) is kept.

if mode == MODE_CRUISE and connected:

seen = husky.seen_ids((GREEN_ID, RED_ID))

if PRINT_DEBUG and seen:

print("COLORS seen:", sorted(seen))

# GREEN -> dodge LEFT (logical -35), recover RIGHT (+50)

if GREEN_ID in seen and not green_latched:

green_latched = True

print("GREEN - pass on the LEFT")

hub.light.on(Color.GREEN)

hub.display.icon(Icon.ARROW_LEFT)

color_dodge(husky, GREEN_ID, GREEN_STEER, GREEN_RECOVER)

baseline_left = None

baseline_right = None

open_count_left = 0

open_count_right = 0

filtered_wall_error = 0.0

last_steering_target = 0

previous_yaw_deg = yaw_left_positive()

recenter_until = clock.time() + RECENTER_MS

hub.light.on(Color.WHITE)

hub.display.icon(Icon.TRUE)

start_drive()

continue

if GREEN_ID not in seen:

green_latched = False

# RED -> dodge RIGHT (logical +35), recover LEFT (-50)

if RED_ID in seen and not red_latched:

red_latched = True

print("RED - pass on the RIGHT")

hub.light.on(Color.RED)

hub.display.icon(Icon.ARROW_RIGHT)

color_dodge(husky, RED_ID, RED_STEER, RED_RECOVER)

baseline_left = None

baseline_right = None

open_count_left = 0

open_count_right = 0

filtered_wall_error = 0.0

last_steering_target = 0

previous_yaw_deg = yaw_left_positive()

recenter_until = clock.time() + RECENTER_MS

hub.light.on(Color.WHITE)

hub.display.icon(Icon.TRUE)

start_drive()

continue

if RED_ID not in seen:

red_latched = False

# ---- opening detection (CRUISE) ----

if mode == MODE_CRUISE:

baseline_left, open_count_left = update_side_reading(

left_mm, baseline_left, open_count_left, OPEN_DISTANCE_MM

)

baseline_right, open_count_right = update_side_reading(

right_mm, baseline_right, open_count_right, OPEN_DISTANCE_MM

)

left_triggered = open_count_left >= SPIKE_CONFIRMATIONS

right_triggered = open_count_right >= SPIKE_CONFIRMATIONS

if left_triggered or right_triggered:

if left_triggered and (

not right_triggered or open_count_left >= open_count_right

):

turn_direction = 1

turn_side_baseline = baseline_left

print("Opening on the LEFT - waiting for front wall")

else:

turn_direction = -1

turn_side_baseline = baseline_right

print("Opening on the RIGHT - waiting for front wall")

front_close_count = 0

wall_return_count = 0

mode = MODE_APPROACH

phase_start_position = drive.angle()

# ---- APPROACH: hold heading until the arc should start ----

if mode == MODE_APPROACH:

opening_side_mm = left_mm if turn_direction > 0 else right_mm

# If the "opening" wall closes up again, it was a false spike:

# cancel the turn and re-detect from CRUISE.

wall_back = (

distance_is_valid(opening_side_mm)

and turn_side_baseline is not None

and opening_side_mm <= turn_side_baseline + WALL_RETURN_MARGIN_MM

)

if wall_back:

wall_return_count += 1

else:

wall_return_count = 0

if (

wall_return_count >= WALL_RETURN_CONFIRMATIONS

and (

not distance_is_valid(front_mm)

or front_mm > TURN_AT_FRONT_DISTANCE_MM + SPIKE_JUMP_MM

)

):

print("Opening closed again - back to CRUISE")

mode = MODE_CRUISE

baseline_left = None

baseline_right = None

open_count_left = 0

open_count_right = 0

continue

start_turn = False

if (

distance_is_valid(front_mm)

and front_mm <= TURN_AT_FRONT_DISTANCE_MM

):

front_close_count += 1

else:

front_close_count = 0

if front_close_count >= FRONT_CLOSE_CONFIRMATIONS:

start_turn = True

elif traveled_mm(phase_start_position) >= MAX_APPROACH_DISTANCE_MM:

# Opening detected but the front wall never came close enough;

# turn into the opening anyway rather than plowing ahead.

print("Approach distance timeout - turning into the opening")

start_turn = True

if start_turn:

turn_target_yaw = wrap_angle(

corridor_yaw + turn_direction * TURN_ANGLE_DEG

)

turn_watch.reset()

mode = MODE_TURN

print(

"TURN %s to %.1f deg"

% ("LEFT" if turn_direction > 0 else "RIGHT", turn_target_yaw)

)

hub.display.icon(

Icon.ARROW_LEFT if turn_direction > 0 else Icon.ARROW_RIGHT

)

continue

# ---- TURN: closed-loop 90 deg gyro arc (never stops mid-run) ----

if mode == MODE_TURN:

turn_error = angle_difference(turn_target_yaw, yaw_deg)

steer_cmd_turn = clamp(

TURN_KP * turn_error,

-MAX_TURN_STEERING_DEG,

MAX_TURN_STEERING_DEG,

)

last_steering_target = apply_steering(

steer_cmd_turn, last_steering_target, MAX_TURN_STEERING_DEG

)

start_drive(TURN_SPEED)

if abs(turn_error) <= TURN_DONE_TOLERANCE_DEG:

corridor_yaw = turn_target_yaw

mode = MODE_POST

phase_start_position = drive.angle()

print("Turn done -> POST")

continue

if turn_watch.time() >= TURN_TIMEOUT_MS:

corridor_yaw = turn_target_yaw

mode = MODE_POST

phase_start_position = drive.angle()

print("TURN TIMEOUT - forcing POST")

continue

# ---- POST: short gyro-straight, then back to CRUISE ----

if mode == MODE_POST:

if traveled_mm(phase_start_position) >= POST_TURN_DRIVE_MM:

turns_completed += 1

print(

"Turns completed: %d / %d"

% (turns_completed, MAX_TURNS)

)

baseline_left = None

baseline_right = None

open_count_left = 0

open_count_right = 0

filtered_wall_error = 0.0

recenter_until = clock.time() + RECENTER_MS

if turns_completed >= MAX_TURNS:

mode = MODE_FINAL

phase_start_position = drive.angle()

print("FINAL: all turns done - driving to the finish")

hub.display.icon(Icon.TRUE)

continue

mode = MODE_CRUISE

print("Back to CRUISE")

continue

# ---- FINAL: settle for FINAL_DRIVE_MM, then stop ----

if mode == MODE_FINAL:

if traveled_mm(phase_start_position) >= FINAL_DRIVE_MM:

stop_drive()

steer_logical(0, wait_done=True)

print("Course complete - stopping")

hub.display.icon(Icon.TRUE)

return

# ---- steering controller + drive ----

# IMPORTANT: this whole block is SKIPPED during MODE_TURN. The TURN

# block above already commands the steering and the drive speed, and

# it runs later in the loop -- if this block ran too it would

# OVERRIDE the turn command every cycle (the cruise controller is

# clamped to 32 deg and uses corridor_yaw, not turn_target_yaw).

# That override is why the car turned the wrong way, turned very

# late, or never turned at all and hit the front wall.

if mode != MODE_TURN:

steer_cmd = 0.0

kp_wall_use = WALL_KP

deadband_use = WALL_DEADBAND_MM

if mode == MODE_CRUISE and clock.time() < recenter_until:

kp_wall_use = RECENTER_WALL_KP

deadband_use = RECENTER_DEADBAND_MM

# heading hold (left-positive). NOTE: the P term is negative on

# purpose -- heading_error > 0 means the car points LEFT of the

# corridor, so it must steer RIGHT (negative) to come back.

steer_cmd = -HEADING_KP * heading_error - YAW_DAMPING * yaw_change

# wall-centering (CRUISE and FINAL only; needs both side walls)

if (

(mode == MODE_CRUISE or mode == MODE_FINAL)

and distance_is_valid(left_mm)

and distance_is_valid(right_mm)

):

raw_wall = (left_mm - right_mm) / 2.0   # + = too close to RIGHT wall

if abs(raw_wall) <= deadband_use:

raw_wall = 0.0

filtered_wall_error += (

raw_wall - filtered_wall_error

) * WALL_FILTER_AMOUNT

steer_cmd += kp_wall_use * filtered_wall_error

last_steering_target = apply_steering(

steer_cmd, last_steering_target, MAX_STEERING_DEG

)

# Speed: in CRUISE, a close front wall with NO opening detected

# means a color block that has not been identified yet -- slow

# down and keep going straight. No turn, no stop.

if (

mode == MODE_CRUISE

and distance_is_valid(front_mm)

and front_mm <= BLOCK_AHEAD_SLOWDOWN_MM

):

start_drive(BLOCK_AHEAD_SPEED)

else:

start_drive()

if PRINT_DEBUG:

print(

MODE_NAMES[mode],

"yaw=%.0f" % yaw_deg,

"f=%d l=%d r=%d" % (front_mm, left_mm, right_mm),

"steer=%d" % last_steering_target,

)

wait(LOOP_TIME_MS)

# ------------------------------------------------------------------ entry

def main():

print("Starting WRO obstacle course ...")

husky = None

connected = False

try:

husky = HuskyLens(HUSKY_PORT, HUSKY_BAUD, HUSKY_POWER_PIN)

connected = husky.knock()

except OSError as err:

print("HuskyLens error:", err)

connected = False

if connected:

print("HuskyLens connected")

hub.light.on(Color.WHITE)

hub.display.icon(Icon.HEART)

else:

print("HuskyLens NOT connected - running blind")

hub.light.on(Color.ORANGE)

hub.display.icon(Icon.SAD)

wait(1000)

try:

drive_course(husky, connected)

except Exception as err:

print("Run aborted:", err)

stop_drive()

steer_logical(0, wait_done=False)

hub.light.on(Color.ORANGE)

raise

main()